M5 PRO modifications

Table 1

| Standard CB frequency | Channels | Frequency range (MHz) | Power Output |
| --- | --- | --- | --- |
| EU | 40FM/40AM | 26.965-27.405 | 1W AM /4W FM |
| CE | 40 FM | 26.965-27.405 | 4W |
| UK+CE* | 40AM/40FM | 27.60125-27.99125 | 4W |
| PL | 40AM/40FM | 26.960-27.400 | 4W |
| IO | 40FM/40AM | 26.965-27.405 | 4W |
| I2 | 34FM/34AM | 26.965-26.955 | 4W |
| DE | 80FM/40AM | 26.565-27.405 | 4W |
| IN | 27FM/27AM | 26.965-27.275 | 4W |

Table 2

| Standard CB frequency | Channels | Frequency range (MHz) | Power Output |
| --- | --- | --- | --- |
| EU | 40FM/40AM | 26.965-27.405 | 4W |
| CE | 40 FM | 26.965-27.405 | 4W |
| UK+CE* | 40AM/40FM | 27.60125-27.99125 | 4W |
| PL | 40AM/40FM | 26.960-27.400 | 4W |
| IO | 40FM/40AM | 26.965-27.405 | 4W |
| I2 | 34FM/34AM | 26.965-26.955 | 4W |
| DE | 80FM/40AM | 26.565-27.405 | 4W |
| IN | 27FM/27AM | 26.965-27.275 | 4W |
| RU | 400 AM/FM | 25.615-30.105 | 4W |
| PC | 50 AM/FM | 25.645-29.895 | 4W |
| PX | 400 AM/FM | 25.610-30.100 | 4W |

Table 3

| Standard CB frequency | Channels | Frequency range (MHz) | Power Output |
| --- | --- | --- | --- |
| AU | 40 AM | 26.965-27.405 | 4W |
| NL | 40 AM | 26.330-26.770 | 4W |
| AM** | 10 AM | 27.680-27.980 | 4W |

*in this band when you are on UK band if keep press the A/F button (3 seconds) the radio pass on CE band and vice versa.

**in this band when you push button EMG(9/19) the radio goes on CH88 (emergency 1)/CH86 (emergency 2).

- How to change "standard CB frequency" in Table 1 - Table 2 - Table 3:

turn on the radio by simultaneously holding down the PTT and AM/FM (mode) buttons.

Then use the UP/DOWN buttons to select the desired option and confirm with PTT.

- How to change "table band":

turn on the radio by simultaneously holding down the PTT, UP and MENU buttons.

Then use the UP/DOWN buttons to select the desired option and confirm with PTT.

TO INCREASE THE POWER OUTPUT:

Keep press DOWN button and PTT during Power on. The display will shows “POWER H”, this mean power output 8W AM/FM.

“POWER L” means default power setting.